# Charge Daddy Command Set

## Communication Transaction Flow

- Build a Command packet (as described below).

- Issue a LockerStatus Command to make sure the board is ready

- If not ready or a status error, do something else?

- Issue the Command Packet

- Wait for Completion (timeout in firmware, but app needs to wait also + a few seconds to allow for some timing slack)

- Check Response Packet

The Pascal code calls a COMSendRaw(out CommandPacket, in Response) function to handle each command transaction.

One of the hardware test programs uses the following Pascal code to perform the serial I/O.  Configuration of the serial port is a separate issue (TBD).

procedure TfrmMain.COMSendRaw(const AOutput: TBytes; var AInput: TBytes);

const

CATTEMPT_MIN = 1;

CATTEMPT_MAX = 10;

CATTEMPT_SLEEP = 50; // ms

var

LCntAttempt: Integer;

LDataLen: Integer;

begin

{$MESSAGE WARN 'changed'}

(* I think this is a Delphi block comment, but I’m not 100% sure

if dmLNCTest.cpCompsLocks.Connected then

try

dmLNCTest.cpCompsLocks.ClearBuffer(True, True);

except

on E: Exception do begin

LogEx(E, 'Clear buffer error.');

Exit;

end;

end; // trye

*)

if NOT dmLNCTest.cpCompsLocks.Connected then

try

dmLNCTest.cpCompsLocks.Open;

except

on E: Exception do begin

LogEx(E, 'Open port error.');

Exit;

end;

end; // trye

dmLNCTest.cpCompsLocks.Write(AOutput[0], Length(AOutput));

///

///  log sent bytes

///

LogOut(AOutput);

LCntAttempt := CATTEMPT_MIN;

repeat

Sleep(CATTEMPT_SLEEP);

LDataLen := dmLNCTest.cpCompsLocks.InputCount;

if LDataLen >= szLNCPacketSizeMinResp then begin

SetLength(AInput, LDataLen);

dmLNCTest.cpCompsLocks.Read(AInput[0], LDataLen);

dmLNCTest.cpCompsLocks.ClearBuffer(True, True);

end;

Inc(LCntAttempt);

until (LCntAttempt >= CATTEMPT_MAX) or (LDataLen > 0);

//  dmLNCTest.cpCompsLocks.Close;

///

///  log received bytes

///

if LDataLen > 0 then

LogIn(AInput)

else

LogErr('NO RESPONSE FROM HARDWARE!');

end;

## Command Packet Definition

Generally, outbound commands are framed by a two byte start prefix (0x7E, 0X1B), an address byte (zero may mean all boards?), the payload, and closed out with a two byte CRC16 value (little endian) and a two byte suffix (0x81, 0xE4).

Start Byte 1 (SOP1): 0x7E

Start Byte 2 (SOP2): 0x1B

Address Byte – matches the drawer/locker; zero may mean all lockers

Command Bytes: (see individual commands below) commands are populated byte at a time, except for 16 bit Charging timer in RestoreChargingREOTT which is big-endian)

CRC16:  2 bytes  ( This is a normal Intel 16 bit word in little endian format)

End Byte 1 (EOP1): 0x81

End Byte 2 (EOP2) 0xE4

## Response Packet Definition

Generally, responses are formatted the same as Command Packets, the response from the controller looks like:

SOP1, SOP2, ADDRESS, 0x05, RXB_01, STATUS, VOLTAGE, STATUS2, 0x34, 0x35, EOP1, EOP2

RXB_01 is a controller firmware flag that appears to report whether the board understood the previous/associated command, but don’t hold me to the exact meaning – I think zero means SUCCESS.

There may be (but uncertain) a CRC16 value before EOP1 rather than the fixed values (0x34, 0x35). The firmware seems to be using (0x34, 0x35) in the place of the CRC.

Success – software is presumably keeping track of most recent activity and Success means the Controller says he’s finished the last operation. E ither way, the LockerStatus Struct reflects the status of that drawer.

Locker Status Structure, 8 or 16 bits reflecting basic condition of the Locker/Drawer in question:

This section is still under construction; Robert and I will try and verify all of this on Monday

DoorOpened: 	BIT 0

TamperDepressed: 	BIT 1

FrontPanelSwitchPressed: 	BIT 2

In4Closed: 	BIT 3

LEDRed: 	BIT 4

LEDGreen: 	BIT 5

LatchRetracted: 	BIT 6

SounderOn: 	BIT 7

REX Status Byte:

DoorIsOpened: 	BIT 0

DoorIsClosed: 	BIT 1

LockTimedOut: 	BIT 2

Tamper: 	BIT 3

FrontPanelSw: 	BIT 4

In4: 	BIT 5

RedIndicator: 	BIT 6

GreenIndicator: 	BIT 7

LatchRetracted: 	BIT 8

Sounder: 	BIT 9

## Command Set Payload Details

### UnlockRentalStart

COMMAND PACKET: SOP1, SOP2, Address, 0x03, 0x61, Timeout (seconds) , CRC16, EOP1, EOP2

RESPONSE PACKET:

### UnlockRentalEnd

COMMAND PACKET: SOP1, SOP2, Address, 0x03, 0x62, Timeout (seconds), CRC16, EOP1, EOP2

RESPONSE PACKET:

### LockerStatus – After each command (unlock, lock, etc.) call LockerStatus to verify it’s completed physical operation.

COMMAND PACKET: SOP1, SOP2, Address, 0x02, 0x02, Timeout(seconds),CRC16, EOP1,EOP2

RESPONSE PACKET:

Generally formatted the same as Command Packets, the response from the controller looks like:

SOP1, SOP2, ADDRESS, 0x05,RXB_01, STATUS, VOLTAGE, STATUS2, 0x34, 0x35, EOP1, EOP2

RXB_01 is a controller firmware flag that appears to report whether the board understood the previous/associated command, but don’t hold me to the exact meaning – I think zero means success

There may be (but uncertain) a CRC16 value before EOP1, the x034, 0x35 values seems to be taking the place of the CRC; firmware doesn’t seem to call the CRC calculation in the current version.

Success – software is presumably keeping track of most recent activity and Success means the Controller says he’s finished the last operation. E ither way, the LockerStatus Struct reflects the status of that drawer.

Status Struct:

DoorOpened: 	BIT 0

TamperDepressed: 	BIT 1

FrontPanelSwitchPressed: 	BIT 2

In4Closed: 	BIT 3

LEDRed: 	BIT 4

LEDGreen: 	BIT 5

LatchRetracted: 	BIT 6

SounderOn: 	BIT 7

### SetLedWithRate

COMMAND PACKET: SOP1, SOP2, Address, 0x03, 0x63, BIT( LedRed | LedGreen | LedBlue | LedWhite | LedHandle), Rate (seconds),CRC16,EOP1,EOP2

RESPONSE PACKET:

### ResetAllToEndOfRental

COMMAND PACKET: SOP1, SOP2, Address, 0x02, 0x64, CRC16, EOP1, EOP2

RESPONSE PACKET:

### RestoreChargingREOTT

COMMAND PACKET: SOP1, SOP2, Address, 0x04, 0x65, Timer (2 bytes Big-Endian?, seconds),  CRC16, EOP1, EOP2

RESPONSE PACKET:

WriteToEEProm – write to controller EEPROM (large payload)

COMMAND PACKET: SOP1, SOP2, Address, 0x0C, 0x66, DATA TBD, CRC16, EOP1, EOP2

RESPONSE PACKET:

### GetFirmwareVersion

COMMAND PACKET: SOP1, SOP2, Address, 0x02, 0x67, CRC16, EOP1, EOP2

RESPONSE PACKET:  CHAR(Major), CHAR(Minor) – ascii characters; the firmware source we have is V3H

### REXUnlock

COMMAND PACKET: SOP1, SOP2, Address, 0x03, 0x01, Timeout(seconds), CRC16, EOP1, EOP2

RESPONSE PACKET:

### REXStatus

COMMAND PACKET: SOP1, SOP2, Address, 0x02, 0x02, CRC16, EOP1, EOP2

RESPONSE PACKET:

### LockerUnlock – uncertain what the difference is with UnlockRentalStart or UnlockRentalEnd; I’m not sure why the hardware would care.

COMMAND PACKET: SOP1, SOP2, Address, 0x03, 0x01, Timeout(seconds),CRC16, EOP1,EOP2

RESPONSE PACKET:

# Issues / TBD

- Response Packets are probably as documented here, but we need to verify them.

- Global commands (Unlock all) may just be iterations of Unlock <Address>, but an address of zero might mean all

- WriteToEEProm payload not examined yet.

- RestoreChargingREOTT probably is used to turn on and off the charging circuit; if the timeout is zero presumably we should turn off the charger. TBD

- What is a “REX” (REX Unlock, REX Status, REX FirmwareVersion)?