SECTION 6

NV200

MANUAL SET

TECHNICAL APPENDICES

NV200 Manual Set – Section 6	2

NV200 MANUAL SET – SECTION 6

| 6. | TECHNICAL APPENDICES | 3 |
| --- | --- | --- |
|  | APPENDIX A – PRODUCT APPROVALS | 3 |
|  | APPENDIX B – TECHNICAL SPECIFICATIONS | 5 |
|  | APPENDIX C – GLOSSARY OF TERMS | 7 |
|  | APPENDIX D – ORDERING INFORMATION | 10 |
|  | APPENDIX E – IDENTIFYING THE NV200 ISSUE NUMBER | 11 |

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

| NV200 Manual Set – Section 6 | 3 |
| --- | --- |

6.	TECHNICAL APPENDICES

APPENDIX A – PRODUCT APPROVALS

CE Marking

The NV200 unit described in this manual set has been designed to comply with the relevant sections of the following Harmonised European Standards:

- EN60950-1:2001

- EN60335-1:2002

- EN60335-2-82:2003

The unit complies with all the applicable essential requirements of the Standards.

RoHS

The following products, identified by the part numbers listed in the table below, are compliant with the European Union Directive 2002/95/EC of the Restriction of the use of certain Hazardous Substances (RoHS) in Electrical and Electronic Equipment.

| Product | Description | Lead free date |
| --- | --- | --- |
| NV200 | Bank Note Acceptor Assembly | All NV200 |

We hereby declare that lead (Pb), mercury (Hg), cadmium (Cd), hexavalent chromium (Cr4-6), polybrominated biphenyls (PBB) and polybrominated diphenyl ethers (PBDE), are not intentionally added to our products in amounts exceeding the maximum concentration values as defined by RoHS regulations (except where the application of any of those substances comes within the scope of the RoHS regulations exempted applications).

All compliant products are clearly marked on the product and/or packaging.

All the information provided in this statement of compliance is accurate to the best of our knowledge, as of the date of this publication being issued.

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

| NV200 Manual Set – Section 6 | 4 |
| --- | --- |

WEEE

The European Union’s directive 2002/96/EC on Waste Electrical and Electronic

Equipment (WEEE) was adopted by the European Council and Parliament in 2003 with a view to improving the collection and recycling of Waste Electrical and Electronic Equipment throughout the EU, and to reduce the level of non-recycled waste. The directive was implemented into law by many EU member states during 2005 and 2006.

Products and packaging that display the symbol (shown left) indicates that this product must NOT be disposed of with other waste. Instead it is the user’s responsibility to dispose of their Waste Electrical and

Electronic Equipment by handing it over to an approved reprocessor, or by returning it to the original equipment manufacturer for reprocessing.

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

| NV200 Manual Set – Section 6 | 5 |
| --- | --- |

APPENDIX B – TECHNICAL SPECIFICATIONS

The information contained here does not form part of a contract and is subject to change without notice. Innovative Technology Ltd operates a policy of continual product development; as such specifications may change from time to time.

Environment:

|  |  | Minimum |  | Maximum |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature |  | +3 °C |  | +50 °C |  |  |
| Humidity |  | 5 % |  | 95 % non condensing |  |  |
| Power Requirements: |  |  |  |  |  |  |
| DC Voltage |  | Minimum | Nominal |  | Maximum |  |
| Absolute limits |  | 10.8 V | 12 V |  | 24 V ** |  |
| Supply ripple voltage |  | 0 V | 0V |  | 0.25 V @ 100 Hz |  |
| Supply Current |  |  |  |  |  |  |
| Standby |  |  |  |  | 400 mA |  |
| Running |  |  |  |  | 1.5 A |  |
| Peak (motor stall) |  |  |  |  | 3 A |  |

WARNING!

Use correct supply voltage

** NOTE: Only the later models of NV200 are capable of using a supply voltage up to 24 V DC. Earlier versions have a maximum voltage of 13.2 V DC.

See Appendix E of this manual (Identifying The NV200 Issue Number) for information on how to identify if your validator supports 24V DC operation.

We recommend that your power supply is capable of supplying 12V DC at 3 A, or 24V DC at 1.5 A - TDK Lambda produces a range of suitable power supplies:

- For 12V operation, use TDK Lambda model SWS50-12. This power supply is available from a variety of suppliers including Farnell (stock code 1184645) and RS (stock code 466-5869).

- For 24V operation, use TDK Lambda model SWS50-24. This power supply is also available from a variety of suppliers including Farnell (stock code 1184646) and RS (stock code 466-5875).

It is recommended that a ferrite cable clamp be used on the cable to the NV200, with the cable having a single turn around the ferrite. The suggested ferrite is as follows:

|  | Manufacturer: | Fair-Rite |
| --- | --- | --- |
|  | Manufacturer Part Number: | 0443166651 |

- Impedance (100MHz @ 1 turn):  225 Ohms

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

| NV200 Manual Set – Section 6 | 6 |
| --- | --- |

Logic Levels:

| Interface Logic Levels |  | Logic Low | Logic High |  |  |
| --- | --- | --- | --- | --- | --- |
| Inputs |  | 0 V to 0.5 V | +3.7 V to +12 V |  |  |
| Outputs (2.2 kΩ pull-up) |  | 0.6 V | Pull-up voltage of host |  |  |
|  |  |  | interface |  |  |
| Maximum current sink |  |  | 50 mA per output |  |  |
| General Specifications: |  |  |  |  |  |
| Note Sizes |  | Minimum |  | Maximum |  |
| Width |  | 60 mm |  | 85 mm |  |
| Length |  | 115 mm |  | 170 mm |  |
| Capacity |  |  |  |  |  |
| Storage |  | 500 or 1000 notes |  |  |  |
| Weight |  |  |  |  |  |
| NV200 |  | 2.9 kg |  |  |  |
| Interface Protocol |  |  |  |  |  |
|  |  | eSSP; ccTalk; SIO |  |  |  |
|  |  | ** MDB; Parallel; Pulse; Binary |  |  |  |

Information

Additional interface required.

** NOTE: Using the NV200 with any of the following protocols will require an additional external interface unit:

MDB; Binary; Parallel; Pulse

See the table below for details of the external interface unit required.

| Interface Protocol | External Interface |
| --- | --- |
| eSSP | --- |
| ccTalk | --- |
| SIO | --- |
| MDB | IF5 |
| Binary | IF9 |
| Parallel | IF10 |
| Pulse | IF15 |

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

NV200 Manual Set – Section 6	7

APPENDIX C – GLOSSARY OF TERMS

| Term | Meaning |  |
| --- | --- | --- |
| A | Ampere |  |
| AC | Alternating Current |  |
| ACK | Acknowledge |  |
| AES | Advanced Encryption Standard |  |
| ASSY | Assembly |  |
| AV | Average |  |
| AWG | American Wire Gauge |  |
| AWP | Amusement With Prizes |  |
| BNV | Bank Note Validator |  |
| ccTalk | Coin Controls Talk |  |
| COMMS | Communications |  |
| CRC | Cyclic Redundancy Check |  |
| DC | Direct Current |  |
| DIA | Diameter |  |
| DIP | Dual Inline Package |  |
| ECB | Electronic Code Book |  |
| EEPROM | Electrically Erasable Programmable Read Only |  |
|  | Memory |  |
| eSSP | Encrypted Smiley® Secure Protocol |  |
| FAQ | Frequently Asked Questions |  |
| GA | General Assembly |  |
| GND | Ground |  |
| Hz | Hertz |  |
| ITL | Innovative Technology Ltd |  |
| Copyright © Innovative Technology Ltd 2013 |  | GA00620-2 |

NV200 Manual Set – Section 6	8

Term	Meaning

LED	Light Emitting Diode

mA	milliampere

max	maximum

MDB	Multi Drop Bus

min	minimum

mm	millimetre

ms	millisecond

MOD	Modified (or Modification)

NV	Note Validator

PCB	Printed Circuit Board

PDF	Portable Document Format

PiPS	Pay-in Pay-out System

PROM	Programmable Read Only Memory

PSU	Power Supply Unit

QTY	Quantity

RAM	Random Access Memory

ROM	Read Only Memory

Rx	Receive

RoHS	Restriction of the use of certain Hazardous

Substances

SIO	Serial Input Output

SSP	Smiley® Secure Protocol

SWG	Standard Wire Gauge

SWP	Skill With Prizes

SYNC	Synchronize

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

NV200 Manual Set – Section 6	9

Term	Meaning

TTL	Transistor Transistor Logic

Tx	Transmit

USB	Universal Serial Bus

V	Volt

V_In	Voltage In

WEEE	Waste Electrical and Electronic Equipment

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

| NV200 Manual Set – Section 6 | 10 |
| --- | --- |

APPENDIX D – ORDERING INFORMATION

The following information is required to order an NV200 validator:

| Product | NV200 | Consists of NV200 validator and cash box chassis |  |
| --- | --- | --- | --- |
| Dataset | Country code and | Alternatively supply details of the currency and |  |
|  | variant | note types you wish to use |  |
| Bezel Size | 82mm; 85 mm | These are the only bezel sizes available |  |
| Bezel Colour | RGB code or text |  |  |
|  | description |  |  |
| Cash Box | 500 or 1000 note |  |  |
|  | capacity |  |  |
| Interface | eSSP; ccTalk; | Using the NV200 with any of the following |  |
|  |  | protocols will require an additional external |  |
|  | SIO; MDB; |  |  |
|  |  | interface unit: |  |
|  | Parallel; Pulse; |  |  |
|  | Binary | MDB; Parallel; Pulse; Binary |  |

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

| NV200 Manual Set – Section 6 | 11 |
| --- | --- |

APPENDIX E – IDENTIFYING THE NV200 ISSUE NUMBER

Early revisions of the NV200 did not allow the use of a 24V DC supply. You need to check the NV200 issue number to see if the validator has 24V compatibility:

Open the NV200 validator lid and check the marking on the PCB where shown in this picture – the marking needs to read PB00266_4 or higher

PCB marking

If the PCB issue (the last digit) is lower than 4, the NV200 validator can only be used with a 12V DC supply.

| Copyright © Innovative Technology Ltd 2013 | GA00620-2 |
| --- | --- |

NV200 Manual Set

MAIN HEADQUARTERS

Innovative Technology Ltd

Derker Street – Oldham – England - OL1 4EQ Tel: +44 161 626 9999 Fax: +44 161 620 2090 E-mail: support@innovative-technology.co.uk Web site: www.innovative-technology.co.uk

BRAZIL suporte@bellis-technology.com.br

CHINA support@innovative-technology.co.uk

GERMANY supportDE@innovative-technology.eu

SPAIN supportES@innovative-technology.eu

UNITED KINGDOM

support@innovative-technology.co.uk

UNITED STATES OF AMERICA

supportusa@bellis-technology.com

REST OF THE WORLD

support@innovative-technology.co.uk

| Copyright © Innovative Technology Ltd 2013 |  | GA00620-2 |  |
| --- | --- | --- | --- |